Приложение

| Комманда | Короткая форма | По умолчанию | Параметры | ## Описание |
| --- | --- | --- | --- | --- |
| RESET | RESET | - | - | Restores the default status for all the commands. |
| RESPTIME | RES | 5 | 5 | Specifies the acknowledgement packet transmission delay. The unit of the parameter is 100 milliseconds. |
| RESTART | RESTART | - | - | Causes the TNC function as if it is switch OFF then ON. |
| RETRY | RE | 10 | 10 | Specifies the number of transmission retries. If packets are not correctly accepted while in connection, a connect request is sent again after the specified number of retries. |
| SENDPAC | SE | $0D | $0D | Specifies a character that forces a packet to be sent. |
| SLOTTIME | SL | 3 | 3 | Specifies the period of random number generation intervals for the PERSIST/SLOTTIME method. The unit of the parameter is 10 milliseconds. |
| TRACE | TRAC | OFF | OFF | When ON, causes the TNC to display all received packets in their entirety. |
| TRIES | TRI | 0 | 0 | Specifies the number of transmission retries programmed in the retry counter. |
| TXDELAY | TX | 50 | 50 | Specifies the time delay between PTT ON and start of transmission. The unit of the parameter is 10 milliseconds. |
| UNPROTO | U | CQ | CQ | Specifies call signs to send a packet in Unprotocol mode. Call 1 is the call sign of the destination. Call2 to call9 are call signs of stations to be digipeated through. |
| XFLOW | X | ON | ON | Causes the TNC to perform software flow control when ON, or hardware flow control when OFF. |

44