1. PROGRAMS

---------------------------------------------------------

The following programs may be used

to download the Russian cyrillic fonts

into Canon Bubble Jet 10e printer:

BJ_RUSM.EXE - for Russian main code table

BJ_RUSA.EXE - for Russian alternate code table

2. PREPARING THE PRINTER

---------------------------------------------------------

Before using the programs to download fonts into

the Bubble Jet printer you must set the DIP switches

to the following configuration:

DIP switch 5 = ON   (Set 2)

DIP switch 8 = ON   (Enable download)

3. USING THE PROGRAMS

---------------------------------------------------------

The programs are called in the following way:

BJ_RUSx [printer_port] [initial_mode]

where:

BJ_RUSx is BJ_RUSM or BJ_RUSA

printer_port is LPT1 or LPT2

parallel port the printer is connected to;

default is LPT1

initial_mode

the printer is switched to this mode after downloading;

the following modes are available:

LQ      = letter quality, 10 cpi

LQ10    = letter quality, 10 cpi (same as LQ)

LQ12    = letter quality, 12 cpi

LQ17    = letter quality, 17 cpi

DRAFT   = draft, 10 cpi

DRAFT10 = draft, 10 cpi (same as DRAFT)

DRAFT12 = draft, 12 cpi

DRAFT17 = draft, 17 cpi

PROP    = proportional

default is LQ

NOTES:

1.  You must turn on the printer before starting

the font downloading programs.

2.  The downloaded fonts remain in printer's memory

until you turn it off; you must reload the fonts

after turning the printer on again.

EXAMPLES:

BJ_RUSM

download Russian fonts (main code table) to LPT1;

the printer is set to LQ mode (10 cpi) after downloading

BJ_RUSM LPT2

download Russian fonts (main code table) to LPT2;

the printer is set to LQ mode (10 cpi) after downloading

BJ_RUSA DRAFT

download Russian fonts (alternate code table) to LPT1;

the printer is set to draft mode (10 cpi) after downloading

BJ_RUSA LPT2 PROP

download Russian fonts (alternate code table) to LPT2;

the printer is set to proportional mode after downloading