Heimsoeth CGA Emulator Documentation

------------------------------------

The Heimsoeth CGA Emulator is a program that emulates the IBM CGA card

in its 320x200x4 and 640x200x2 modes on a Hercules compatible graphics

card. It can be started by entering the command 'EMU' from the DOS

command line prompt. If all you want is a simple CGA emulation then you

can stop there. However the EMU can also be used to reconfigure

your Hercules card for other resolutions.

We do assume knowledge of the 8086 hardware architecture and hex

notation for those who dare to go furthur. In the following numbers

are decimal unless they are 32 bit segment:offset address or they

are followed with a small "h", then they are hexadecimal.

To understand how the emulator works it is important to understand the

differences between the CGA and the Hercules graphics card. These cards

are similar in that they both use the Motorola 6845 CRT controller

to display banked memory starting at address B800:0000. The differences

are:

- The CGA's 6845 ports are accessed via port 3B4h, the Hercules

via port 3D4h.

- The CGA displays 2 banks of graphics memory located 2000h bytes

apart at B800:0000 and BA00:0000, the Hercules up to 4 banks located

2000h bytes apart at offsets 0h, 2000h, 4000h and 6000h.

- The CGA works with the IBM Color display monitor, the Hercules

with the IBM monochrom monitor. These have the following specs:

Color   Monochrome

Screen Refresh rate (Hz)          60           50

TTL compatible horizontal scan rate (kHz)      15.750       18.432

Maximum pixel bandwith (MHz)      14.381       16.257

Method of operation

-------------------

The emulator is a TSR program that changes:

- the video interrupt set mode command (interrupt 10h function 0).

- the timer interrupt to copy the memory banks.

- the equipment flag byte (at address 0:0410h) to tell programs

that a CGA is present.

EMU parameters

--------------

The EMU can be configured with the following paramters:

Bank copy parameters:

-B1:nn (1024) = copy nn words from bank 2 to 3 per timer tick

-B2:nn (   0) = copy nn words from bank 1 to 4 per timer tick

6845 register parameters:

Horizontal timing:

-H1:nn (  53) = Num. chars per row minus 1

-H2:nn (  34) = Num. visible chars per row

-H3:nn (  30) = Pos of first char during sync minus 1

-H4:nn (   7) = Num. chars during sync per row minus 1

Vertical timing:

-V1:nn ( 121) = Num. rows minus 1

-V2:nn (   2) = Num. scans in addition to number of rows

-V3:nn ( 100) = Num. visible rows

-V4:nn ( 115) = Row number to begin retrace

Other:

-I:nn  (   2) = 1=interlace, 2=normal

-N:nn  (   2) = Num. scans per row minus 1

-S1:nn (   0) = First scan where cursor will overlay a char

-S2:nn (   2) = Last  scan where cursor will overlay a char

-NT           = Do NOT use timer tick (default is used)

-DE           = Deactivate installed EMU

The usage of these parameters is explained more fully in the following.

On the 6845 data sheet parameters R0-R3 corespond to H1-H4, R4-R7

to V1-V4, R8 to I, R9 to N, S1-S2 to R10-R11. They can be obtained

from the EMU program by inquiring EMU with a question mark or

two.

Configuring EMU

---------------

Three considerations are important in configuring EMU, they must be

considered simulataniously. They are Monitor timing, screen resolution

and bank copying.

Monitor Timing

--------------

The horizontal scan rate, the screen refresh rate, and the total pixel

bandwidth must be compatible with your monitor (presumably the

monochrome monitor mentioned above). Some monitors are more tolerant

with their parameters than others, you'll have to experiment and see.

Try not to get too far out of synch, it is not good for the card or the

monitor. The variables H1, V1, V2 and N refer the the above EMU

input parameters.

Constraint 1:

pixPerChar       =       16                (in graphics mode)

timePerChar      = 0.000001                (in graphics mode)

charsPerRow      =     H1+1                (input parameter)

scanTime         = timePerChar*charsPerRow

horScanRate      = 1/scanTime              (must be around 18.4 k)

Constraint 2:

scansPerRefresh  = ( (V1+1)*(N+1) ) + V2           (input parameters)

refreshTime      = scanTime*scansPerRefresh

refreshRate      = 1/refreshTime           (must be around 50.0)

Constraint 3:

pixPerScan       = pixPerChar*charsPerRow

totalPixels      = scansPerRefresh*pixPerScan*refreshRate

pixBandWidth     = 1/totalPixels           (must be less than 16.2 Mega)

The first two constants are always the same for a Hercules card, thus

the pixel rate always works out to 16 Megahertz (the input parameters

cancel out). Constraint 3 is thus always satisfied.

The first constraint rather directly implies that H1 be 53. The actual

horizontal pixels displayed is determined by H2 (see below), H2 must be

less than H1. H1, H2 and H3 are all 8 bit registers.

The second constraint is a bit more complicated. V3 is the parameter

that determines how many lines are displayed, V3 must be less than than

V1, V2 is a fine adjustment parameter. V1,V3 and V4 are 7 bit registers,

thus their largest value is 127. V2 is a 5 bit register thus it's

largest value is 31.

H3 adjusts the right/left margins on the screen, H4 should be left

alone. V4 adjusts the top/bottom margins. S1 and S2 I haven't played

with.

For more information on these parameters see the 6845 data sheet.

Screen Resolution

-----------------

The screen resolution is determined by H2, V3 and N.

- Horizontal Resolution

The number of horizontal pixels is H2*16.

Notice that H2 must be less than H1.

- Vertical Resolution

The number of vertical pixels is V3*(N+1).

N can be 0, 1, 2, or 3.

V3 must be less than V1.

- Addressing the pixels

The pixel address is the sum of a base address, a horizontal and

a vertical component. The base address is always B800:0000. The

horizontal byte component is always X div 8, the bit number is

7-(X mod 8). The vertical component is calculated differently

depending on the value chosen for N, and H2. Let XBYTES = H2*2,

then we have the following cases:

N=0 - YByte =  y*XBYTES

N=1 - YByte = (y mod 2)*2000h + (y div 2)*XBYTES

N=2 - YByte = (y mod 3)*2000h + (y div 3)*XBYTES

N=3 - YByte = (y mod 4)*2000h + (y div 4)*XBYTES

- Example:

If H2 is 40 (640 horizontal pixels), V3 = 100, and N=2

(EMU's default) then XBYTES is 80 (50h) and the pixel located

at (x,y)=(9,4) is at address

base        hor.component      vert.component

B800:0000 +     x div 8     + (y mod 3)*2000h + (y div 3)*XBYTES

B800:0000 +        1        +     1    *2000h + 1*50h

B800:2051 is the byte address, the bit number is (7-(x mod 8)) is 6.

Thus the bit may be set by OR'ing the byte at B800:2051 with

the value 40h.

Bank Copying

------------

EMU grabs the timer interrupt for the purpose of copying memory banks if

so desired. This bizarre and innovative procedure is useful for filling

the holes that arise from some CGA software. If this feature is not

desired it can be turned off with the -NT flag, in this case the timer

interrupt is left alone.

The B1 and B2 parameters cause copying from

bank2 (B800:2000-B800:3fff) to bank3 (B800:4000-B800:5fff) and

bank1 (B800:0000-B800:1fff) to bank4 (B800:6000-B800:7fff)

respectively. The second bank copy is offset one row for coherence. In

both cases 2000h bytes will be copyed, the parameters determine how many

bytes per tick. In general the higher values will result in a trade-off

of performance for funny dynamic "afterglow" effects as graphics are

written and not promptly copied to the other bank.

General Hints and Suggestons

----------------------------

EMU won't work with a lot of software that checks to see if the

CGA hardware is there before they go on. In general this can

easily be "reverse engineered" with a debuger and a bit of

imagination. Common ways of verifying a CGA's presence include:

- Seeing if changeable memory is at B800:0000 in text mode,

on the CGA it is, on the Hercules not.

- Seeing if the values at port 3BA are changeing, this

indicates whether the CGA video retrace is active or not.

- Interogating the hardware equipment byte at 0000:0410

either directly or through INT 11h.

Turbo Basic requires activation with the EMU.BAS routine.

If a second EMU is loaded over the first, it will deactivate the

former EMU and then proceed. Thus you can experiment with the various

parameters without (often) rebooting by simply loading it over and over.

Since it occupies less than 1K when installed you can try out a lot

of configurations before rebooting.

It would be nice to get a CGA emulation with N=1 and no bank copys,

however with V1=7Fh and V2=1Fh and N=1 you still get a refresh rate of

around 62 Hz and most monochrome monitors won't sync on that. But some

will so try it:

EMU /V1:$7f /V2:$1F /N:1 /NT

There is a lot one can do with the EMU and we would like to hear about

any interesting applications. Write us or tell the hotline:

Heimsoeth Software GmbH and Co.KG

Fraunhoferstr. 13

8000 Muenchen 5

West Germany

Hotline:  Tele - (089)-264060   14:00-17:00 Mon.-Fri.

Bibliography

------------

Motorola SY6845R CRT Controller Data Sheet

IBM XT Technical Reference Manual

Monochrome Graphic/Printer Card User's Manual