RS232 - Asynchronous Communications Port Monitor - Version 2.03

RS232 is a memory resident program that displays RS-232 signals for com1

or com2.  It is intended to enhance internal modems by supplying

software "lights" similar to those found on most external modems.

Command Format: RS232 [   | ? | 1 | 2 ]

where [ ... ] denotes a

choice of the following:

blank - Displays this output before installed

Disables/enables display after installed

? - Displays this output anytime

1 - Installs resident portion of RS232 for COM1

2 - Installs resident portion of RS232 for COM2

No method of removing this program from memory has been implemented.  A

boot is required.

The output of this monitor program is displayed in the upper right hand

corner of the display.  From left to right the meaning of the eight

character display is:

T R R C R C T S

| | | | | | | |____ data (S)et ready

| | | | | | |______ data (T)erminal ready

| | | | | |________ (C)arrier detect

| | | | |__________ (R)ing indicate

| | | |____________ (C)lear to send

| | |______________ (R)equest to send

| |________________ (R)eceive data

|__________________ (T)ransmit data

If any signal is off, its position will show a period.

Compatibility:

RS232 has been tested with IBM and Hercules monochrome and IBM color

graphics video boards.  It has been used with an IBM EGA for a limited

time with no ill effects.  Modems that have been tested with RS232 are

Hayes 1200B, Hayes 300 and Ventel Half Card.  RS232 runs on the author's

Fido node and causes no problems for the Fido software running on an IBM

PC.

Copyright (c) 1986 William H. Beers, IV                 Page - 1

All Rights Reserved                                  23 Dec 1986

RS232 - Asynchronous Communications Port Monitor - Version 2.03

Program limitations:

The status bits used for Receive and Transmit Data get cleared too fast

by the communication programs to be of much use.  Those signals

therefore don't appear much of the time even though data is being

received and transmitted.

Update History:

2.01 - April 86 - Original "working" version.

2.02 - Aug   86 - Internal clean up.

2.03 - Dec   86 - Fix lost characters when running with ProComm.

Added code to RS232 to send EOI to the 8259

instead of waiting for int 8 to do it when

RS232 iret'd from int 1Ch.

Distribution rights:

Permission is granted to freely distribute this code, but not

for profit, provided that this notice, the following disclaimer, and

this documenation file are included in their entirety and without

modifications of any sort or nature.  Commercial use of this work,

without the expressed written consent of the author, is specifically

prohibited.

The author's address:

William H. Beers, IV

1136 Bliss Drive

St. Louis, MO 63137

Sysop, Fido 100/240, FidoNet mail only.

Disclaimer:

This prgoram is provided "as-is" without warranty of any kind,

either expressed or implied, including, but not limited to the implied

warranties of merchantability and fitness for a particular purpose.  The

entire risk as to the results and performance of the program is assumed

by the user.  Should the program prove defective, you (and not the

author) assume the entire cost of all servicing, repair, or correction.

The author or anyone else who has been involved in the creation,

production or delivery of this program shall not be liable for any

direct, indirect, consequential or incidental damages arising out of the

use or inability to use this program.

Despite all the above, the author would appreciate having bugs

reported via FidoNet to SYSOP on node 100/240 or via US Mail.