Implementation note A804X V1.80

===============================

Subject: JMPs and CALLs over memory banks.

The Micro-Series software is designed to check for invalid operations in

a higher extent than is usual.  In most cases this is a benefit to the user,

but in some rare cases it may cause some problems like complaints from the

assemblers.   A problem reported by some users is the fact that the A804X

assembler flags a JMP or CALL from one bank to another as "expression out of

range".   This is also the correct action by the assembler since it is a

special situation involving two instructions (SEL MBx and JMP/CALL).

The following code shows the recommended way of dealing with bank-crossing

instructions:

1  0800           BANKSW  =       800H    ; PC-bit 11

2  0000                   ORG     0       ;Bank #0

3  0000           Zero_bank:

4  0000 F5                SEL     MB1

5  0001 74B8              CALL    Bank_one XOR BANKSW

6  0003 1400              CALL    Zero_bank

7

8  0BB8                   ORG     3000    ; Bank #1

9  0BB8           Bank_one:

10  0BB8 83                RET

11

12  0BB9 E5                SEL     MB0

13  0BBA 1400              CALL    Zero_bank XOR BANKSW

14  0BBC 00                NOP

15  0BBD 64B8              JMP     Bank_one

Note that only in the case you cross a bank you have to add XOR BANKSW to the

JMP/CALL argument.