Dallas Semiconductor

DS5000T Evaluation Kit                                 DS5000TK

FEATURES

Allows immediate evaluation of  any DS5000 series device in an

existing application

Supplied with DS5000T 32, software diskette,  DS5000 User's

Guide, In System Loader hardware

Supports in-system serial downloading of DS5000T from an IBM PC

host

DS5000T supports all functions of DS5000 with addition of real-

time clock

Downloads/verifies Intel Hex absolute object files residing on

IBM PC

User-friendly software prompts user for required system

configuration information

Supports serial download rates up to 9600 bps

Requires no support circuit overhead on target system

DESCRIPTION

The DS5000TK is a development support system which is designed to

allow immediate evaluation of the DS5000T Time Microcontroller in

a system application.  The DS5000TK is an upgraded version of the

DS5000K.   Since  the  DS5000T  performs  all  of  the  functions

associated  with the DS5000 Soft Microcontroller,  it may also be

used for evaluation of any of the versions of a DS5000 for a  new

or existing design.

Materials provided with the kit include a DS5000T with 32  Kbytes

of RAM,  full documentation on the DS5000 and DS5000T,  In System

Loader  serial download hardware,  and KIT5K,  which is a control

software program for the IBM PC.  Also supplied are demo routines

which  exercise  the  built-in real-time clock  function  of  the

DS5000T.

The  In System Loader hardware allows application software to  be

loaded into the DS5000 series device while it is connected to the

target  system,  eliminating the need for removal of  the  device

1

when  reprogramming  is  required.     KIT5K is  a  user-friendly

software package which controls the loading process to the DS5000

while it is installed into the In System Loader hardware.   KIT5K

provides  a  high-level  user interface to the  DS5000T  via  its

Serial Load Mode.  When the Program command is executed, the user

is  walked through a series of system configuration questions  so

that  the DS5000T can be properly initialized before  downloading

takes place.  Parameters such as the device's Program/Data Memory

mapping and Software Encryption operation are initialized in  the

proper  order  in  this  fashion.    KIT5K  manages  all  of  the

communication with the DS5000T during the downloading process, so

that  the  details  of the serial download operation  can  remain

transparent to the user.

With  the  Evaluation Kit,  the user can  quickly  configure  the

DS5000T  for operation in the target system.   This configuration

can  be performed without detailed knowledge of the operation  of

the  DS5000's Serial Load Mode.   The DS5000T Evaluation Kit  not

only  serves as a first-time evaluation system for the DS5000  or

the  DS5000T,  but  also performs the equivalent function  of  an

EPROM programming system throughout the prototyping phase of  the

design  cycle.  If the  user need to program a DS2250, an adaptor

which plugs into the fixture is available.

IN SYSTEM LOADER

The  purpose  of  the In System Loader hardware  is  to  serially

download the DS5000T on command from the KIT5K software in such a

manner  that it will be transparent to the hardware on the target

system.   The  In System Loader hardware illustrated in Figure  1

consists  of  an RS232 cable that connects to the  RS232  Fixture

which  houses the appropriate interface circuitry and provides  a

40-pin  Zero-Insertion-Force  socket  for either  the  DS5000  or

DS5000T.  The Fixture in turn attaches to the 40-pin target cable

which  connects  to  the  microcontroller socket  in  the  target

system.   The  hardware  provides  the mechanism  for  the  KIT5K

software  to  take control of the DS5000T via  the  RS232  cable,

place  the device in its Serial Program Load Mode,  and  transmit

new software to the device.

The  main  components  of  the  In  System  Loader  hardware  are

described in detail below:

2

Dallas

Name           Part #              Description

RS232 Connector     DS9062-6  Adaptor with cable.  Adaptor

provides DB25  female connector for

connection to an RS232C IBM PC COM

port on one side and RJ11 female on

the other side.  Cable carries

RS232 signals required by the kit

with two male RJ11 jacks on either

end.  (Note that although this

cable resembles an ordinary

telephone cable, it is not the

same, and a telephone cable will

not work properly.)

RS232 Fixture       DS9060    RS232 serial interface for DS5000.

Provides RJ11 female for RS232

signal connection, 40-pin DIP IC

socket for DS5000, and 40-pin PC

edge connector for connection to

target cable.

Target Cable        DS9061-5  40-pin adapter cable which connects

the 40-pin edge connector on the

RS232 Fixture to the target system

microcontroller socket.

KIT5K OPERATION

KIT5K  is  the  software environment supplied  with  the  DS5000T

Evaluation  Kit.   It provides a high-level interface for loading

application   software  to  the  DS5000T  or  for   setting   its

configuration  parameters  via the Program  command.    For  more

advanced  users,  KIT5K provides a number of commands which allow

individual manipulation of the DS5000's resources.   For example,

these  commands  allow  the direct  initialization  of  the  MCON

register,  loading of the 40-bit Encryption Key word, and setting

and  clearing of the Security Lock.   In addition,  an individual

memory  location  examine and change capability  is  provided  to

allow  patches to be made to the application software.   When the

loading  operation is completed,  the device can be  released  on

command from the PC to run the application software.

For Users interested in serial communication applications, the

latest version of KIT5K (Version 3.1) includes a dumb  terminal

emulator. This code allows  the user to run  applications which

require serial commands, and to send such commands from the PC.

The serial communication  uses  the  target  system  time base,

allowing flexibility  in  baud  rates.  The  terminal  emulator

simplifies    applications    which   use   the  Dallas  serial

communication  software  module.  Many  of the KIT commands are

available while in terminal emulation.

3

After KIT5K has been  invoked  and the prompt has been displayed

(as  described  above),  commands  may be  entered  by the user.

KIT5K operates either in interactive mode, or in batch mode. The

user will not see any of the communication between KIT5K and the

DS5000T (except when debug is on). The following is a summary of

commands recognized by the KIT5K software.

KIT MODE

cd      Change to another directory or show the default directory.

cls     Clear screen

com     Specify the COM port for the In System Loader hardware.

dir     List the default directory or specified path.

display List  Embedded RAM contents in debug format.

do      Execute a list of KIT5K commands from a file.

dos     Enter DOS mode temporarily

dtr     Toggles the DTR line to switch between run and load

dump    Dump Embedded RAM in Intel Hex to a file.

edit    Individual examine/change Embedded RAM bytes in DS5000.

exit    Exit the KIT5K program; return to MS-DOS.

fill    Fill Embedded RAM with a constant value.

help    Describe the function and syntax of KIT5K commands.

key     Load  the  40-bit Encryption Key word.

load    Load  Embedded RAM from an Intel Hex object  file.

lock    Set the Security Lock on the DS5000.

logoff  Disable logging of KIT5K commands.

logto   Log KIT5K commands to the specified file.

mcon    Set the  MCON register with a specified value.

out1    Toggles a PC/AT bus signal for use by PC based target systems

partition Set the Partition Address with a specified value.

pgmode  Place the in its Serial  Load Mode.

program Program the DS5000 automatically with a configuration file.

quit    Same as exit; leave the KIT5K program; return to MS-DOS.

range   Set the Range Address to 8K or 32K.

reload  Sets the PC UART reload value for flexible baud rate selection

run     Disable Serial Load Mode; begin execution.

speed   Specify the serial baud rate to be used during loading.

status  Display status.

term    Switch to terminal mode, see below

type    Type the requested filename to the screen.

unasm   Unassemble the program memory to view code

unlock  Clear Security Lock.

verify  Verify Embedded RAM with the specified Intel Hex file.

Terminal Mode

capture Capture RAM to memory, and allow uploading to a file

cd      Change directory

cls     Clear screen

com     Set the com port for communication

dir     Read directory

dos     Enter DOS mode temporarily

dtr     Toggle the DTR line

exit    Exit Kit

help    Describe the functions and syntax of terminal emulator

kit     Switch to kit mode

logoff  Stop logging commands to a file

4

logto   Log commands to a file

nosnow  Disable snow checking for CGA screens

out1    Toggles a PC/AT bus signal for use by PC based target systems

pgmode  Program the DS5000 automatically from a configuration file

quit    Leave KIT5K

reload  Sets the PC UART reload value for flexible baud rate selection

run     Run a DS5000 program

send    Send a file to the DS5000 via the serial bus

snow    Enable snow checking for CGA monitors

speed   Select the baud rate

type    List a DOS file.

After  KIT5K has been invoked and the prompt has  been  displayed

(as described above), commands may be entered by the user.  KIT5K

operates either in interactive mode,  or in batch mode.  The user

SYSTEM REQUIREMENTS

The  Evaluation Kit requires an IBM PC or compatible with DOS 2.0

or later and at least 128K bytes of memory.  In addition an RS232

port must be available which is configured as COM1 (03F8H,  IRQ4)

or  COM2  (02F8H,  IRQ3).   Displays which are supported  include

monochrome, color graphics (CGA), or enhanced graphics  (EGA Mode

3).

Power (+5V) must be supplied to the RS232 fixture from the VCC

pin of the target system via the Target Cable (see below).

ELECTRICAL SPECIFICATIONS

Operating Temperature Range             0 to +50 degrees C

System Power Supply Requirements        5V @ 100 mA max; from

Target System 50 mA

typical (DS5000 installed

in target system; no load

on port pins, PSEN*, ALE)

INTERFACE

Connectors:                             25-pin RS232 'D' type to

RJ11 jack adapter

RJ11 on RS232 Fixture

40-pin card edge (0.1"

centers) on RS232 Fixture

40-pin edge connector  on

Target Cable

40-pin DIP plug on Target

Cable

DS5000TK SOFTWARE DISKETTE

The  software diskette supplied with the DS5000TK includes  KIT5K

as  well as a set of programs which work together to allow you to

read  and  set the date and time in a DS5000T and to  compare  it

with  the clock in your IBM or IBM-compatible PC.   All of  these

programs are summarized below:

5

1. KIT5K.EXE        This is the utility program which is used for

programming the DS5000T. It can be used to

read programs to and from the DS5000T,

examine the contents of program memory, set

the range for 8K or 32K, and set or clear the

program memory encryption key.

2. TEST.HEX         This is a short file of text which can be

used to confirm that the KIT5K program can

load data into the DS5000T successfully.

3. DEMO.BAT         This is a batch file which automatically

loads the DS5000T with a program to read the

time and then runs a program in the PC to set

or display the time sent back by the DS5000T.

4. DEMODS5T.CMD     This is the command file which directs the

KIT5K program to load the DEMODS5T.HEX

program into the DS5000T.

5. DEMODS5T.HEX     This DS5000T program responds to commands

received over the serial I/O port to send or

receive the date/time information between the

embedded clock/calendar in the DS5000T and

the serial I/O port.  This allows an external

computer to access to the date/time

information.

6. DEMODS5T.LST     This is a listing of the program DEMODS5T.HEX

described above.

7. DEMODS5T.SRC     This is the source code for the program

DEMODS5T.HEX described above.

8. DEMODS5T.EXE     This is a PC program to test the performance

of a DS5000T by relaying date/time

information between it and the PC and

comparing it against the PC clock.  This

program requires that DEMODS5T.HEX be

installed in the DS5000T, and that the

DS5000T be installed in a DS9060 RS232

Fixture with its serial selection switch in

the B position and connected to one of the

COM ports of the PC.

9. DEMODS5T.PAS     This is the Pascal source code for the

program DEMODS5T.EXE described above.

10. SAMPLE.HEX      This program responds to commands received

over the serial port to set the date and time

information in the DS5000T

11. SAMPLE.LST      This is the listing of the program SAMPLE.HEX

described above.

12. SAMPLE.SRC      This is the source code for the program

SAMPLE.HEX described above.

6

13. KIT.HLP        Help file for KIT mode.

14. TERM.HLP       Help file for terminal mode

INSTALLATION INSTRUCTIONS

To  install the In System Loader hardware,  the user should first

turn power off to the target system.   Next, the RS232 Cable with

the 25-pin DB25 adapter should be connected to either the COM1 or

COM2 port of the IBM PC.  Note that although this cable resembles

an ordinary telephone cable,  it is not the same, and a telephone

cable  will  not work properly.   The RJ11 plug  should  then  be

inserted into the RJ11 jack on the RS232 Fixture.   Finally,  the

40-pin   Target  Cable  should  be  connected  between  the  edge

connector  of  the  RS232 Fixture and the 40-pin  socket  of  the

target  system.   The user should check to insure that the switch

on the RS232 Fixture is in the "A" position for initial checkout.

The KIT5K software is shipped on a 5 1/4" flexible diskette.   As

a  precaution,  a duplicate copy of the diskette should  be  made

before  attempting to run the software on the PC.   The  software

may be executed directly from diskette,  or it can be copied over

to a hard  disk drive and executed from there.

INITIAL CHECKOUT

KIT5K  is  supplied  with a file called "TEST.HEX"  that  may  be

downloaded  to the DS5000T to verify that the Evaluation Kit  has

been  installed correctly.   Below is a checkout procedure  which

downloads  the "TEST.HEX" file to the DS5000T and initializes the

device's   configuration  parameters.    This    procedure   also

demonstrates  the command sequence which is likely to be the  one

most often used to serially download new application software.

After performing the installation procedure described above,  the

initial  checkout procedure can be started by executing the KIT5K

software.  Execution of KIT5K can be invoked  by typing

>kit5K<CR>

following  the  prompt.   The  software  will  respond  with  the

following:

Dallas Semiconductor DS5000 Evaluation Kit Software

Kit5K - Version x.x

Copyright (C) 1987, Dallas Semiconductor Corporation

kit5K>

At  this point,   KIT5K is ready to accept commands.   The  first

action which should be taken is to specify the COM port to  which

the  In System Loader hardware is connected.   Assuming that this

is COM 1 the user would enter the following:

kit5K> com 1 <CR>

KIT5K  may  now  communicate with  the  DS5000's  on-chip  serial

loader.   In order to download the file 'TEST.HEX' to the DS5000,

the user should type:

7

kit5K> program test.hex <CR>

The  user  will  then  be prompted with  a  series  of  questions

regarding  the configuration of the DS5000.   By entering a  <CR>

character for all of these,  the displayed default values will be

assigned as shown below:

Configuration:  Press return for default value

Range (8000) =  <CR>

Partition Address (1000) = <CR>

Encryption (No) <CR>

Locking (No) = <CR>

Begin (0000) = <CR>

End (7FFF) = <CR>

Verify (Yes)=<CR>

After  this  sequence has been completed the software  will  echo

back  the  selected configuration and program the  DS5000  series

device as follows:

Configuration Contents:

Range:  8000

Partition Address:  1000

Encryption:  No

Locking:  No

Begin:  0000

End:  7FFF

Verify:  Yes

Unlocking...

Clearing MCON...

Setting Range...

Setting Partition

Loading...

Verifying...

kit5k>

The    above    response   from   KIT5K   signifies   that    the

configuration/loading operation was completed successfully.   The

user may verify this by executing the command "DISP 0 80" to show

the test data in the DS5000T memory.  This is shown below:

kit5K> disp 0 7F<CR>

0000  01 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00   ................

0010  00 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00   ................

0020  54 45 58 54 20 46 4F 52 - 20 44 49 53 50 4C 41 59   TEXT FOR DISPLAY

0030  20 50 55 52 50 4F 53 45 - 53 2E 20 41 42 43 44 45    PURPOSES. ABCDE

0040  46 47 48 49 4A 4B 4C 4D - 4E 4F 50 51 52 53 54 55   FGHIJKLMNOPQRSTU

0050  56 57 58 59 5A 20 61 62 - 63 64 65 66 67 68 69 6A   VWXYZ abcdefghij

0060  6B 6C 6D 6E 6F 70 71 72 - 73 74 75 76 77 78 79 7A   klmnopqrstuvwxyz

0070  20 31 32 33 34 35 36 37 - 38 39 30 20 30 98 FD C2   1234567890 0....

8

Following the execution of the Program command shown above, KIT5K

automatically created a configuration file with the same name  as

the  specified .HEX file,  but with an extension of  .CFG.   This

file is used to save the DS5000 configuration parameters that the

user specified during the first time that the Program command was

executed  for  the specified .HEX file.

During  future  Program  command  operations,  the  associated

configuration file will be used  to  initialize the parameters

automatically,  without any intervention  by  the user.

If modification  of  the  previously  selected  parameters  is

desired,  the  user  should  select  the  /edit  function when

invoking the Program command as shown below:

kit5K> program test.hex  /edit<CR>

COMMAND LINE SYNTAX

Single-letter ASCII character strings are recognized as commands.

Commands  will  not be processed until an entire command line  is

entered and terminated with a <CR>.   Since a command line is not

processed   until a  <CR> is entered,  it may be edited with  the

delete key which will do a destructive delete to the screen.

Only  legal characters will be echoed back to  the  screen.   The

legal      characters    are:     0123456789    <:>,     <space>,

ABCDEFGHIJKLMNOPQRSTUVWXYZ,     abcdefghijklmnopqrstuvwxyz,   and

<delete>.   Backspace  characters (<BS>) are converted to  delete

characters.  The horizontal tab character is converted to  space.

In  most  commands,  arguments  are  represented  by  hexadecimal

numbers.   A  hexadecimal  number is any sequence of  hexadecimal

characters.   A  hexadecimal character may be a digit,  0 through

9, or one of the  letters A through F.  A byte will always be the

right-most two digits of a hexadecimal number.   For example, the

following hexadecimal numbers will result in the following bytes:

A               ->      0AH

AB              ->      0ABH

ABC             ->      0BCH

ABCD            ->      0CDH

An  address  will  always  be the right-most  four  digits  of  a

hexadecimal  number.    For  example  the  following  hexadecimal

numbers will result in the following addresses:

A               ->      000AH

AB              ->      00ABH

ABC             ->      0ABCH

ABCD            ->      0ABCDH

ABCDE           ->      0BCDEH

In the other cases, arguments are represented by decimal numbers.

In  commands which normally accept hexadecimal numbers  as  their

arguments,  decimal  operation may be invoked by typing a  period

(.) after the entered value.

9

DETAILED COMMAND SUMMARY

KIT MODE

Cd directory_name

The default directory is set to the argument.

Example:

kit5K> cd \ws

C:\WS

kit5K>

Cls

Clear the monitor screen

Com (1 | 2)

Selects which PC communication port to use.   DOS puts the  port

base numbers into  a vector area in  memory.  This program  will

attempt to use either of these two addresses (there may  be zero

or one value here) after the base  has  been checked against the

hexadecimal base values of 3F8 and 2F8.

Example:

kit5K> com 1

kit5K>

Dir (filename | wildcards)

A list of the file names in the directory are displayed.

Example:

kit5K> dir

RUNME.EXE      READ.ME       KIT5K.EXE

TEST.HEX       DEMO.BAT      DEMODS5T.CMD   DEMODS5T.EXE  DEMODS5T.HEX   DEMODS5T.LST

DEMODS5T.PAS   DEMODS5T.SRC  SAMPLE.HEX     SAMPLE.LST    SAMPLE.SRC

kit5K>

Display  beg-addr, end-addr  >filename

Data is displayed as in MS-DOS debug. The optional arguments are

interpreted as hexadecimal  numbers as a default.  The beginning

and ending addresses are positional and will default to zero and

the  current  range  value, if not specified. The "greater than"

sign signifies that output is to go to the specified file instead

of to the screen.

10

Example:

kit5K> dis 0 80

0000  01 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00 ................

0010  00 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00 ................

0020  54 45 58 54 20 46 4F 52 - 20 44 49 53 50 4C 41 59 TEXT FOR DISPLAY

0030  20 50 55 52 50 4F 53 45 - 53 2E 20 41 42 43 44 45 PURPOSES. ABCDE

0040  46 47 48 49 4A 4B 4C 4D - 4E 4F 50 51 52 53 54 55 FGHIJKLMNOPQRSTU

0050  56 57 58 59 5A 20 61 62 - 63 64 65 66 67 68 69 6A VWXYZ abcdefghij

0060  6B 6C 6D 6E 6F 70 71 72 - 73 74 75 76 77 78 79 7A klmnopqrstuvwxyz

0070  20 31 32 33 34 35 36 37 - 38 39 30 20 30 98 FD C2 1234567890 0...

0080  98                      -                           .

kit5K>

Do filename

The commands in the filename are executed as a script until the

end of file is encountered, or until an error occurs.

Example:

kit5K> type script

fill ff 0 2f

display 0 2f

kit5K> do script

fill ff 0 2f

display 0 2f

0000  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0010  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0020  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

kit5K>

Dos

Takes the user  to the DOS prompt to execute commands, without

actually  leaving  KIT5K.   When  DOS  commands  are  complete,

return to KIT by typing "exit".

Example

kit5k > dos

Type Exit to return to Kit5K

DOS Message

c:\kit > copy \test.hex  \kit\prog,hex

c:\kit > exit

Welcome back to KIT

11

Dtr

Toggles the status of the DTR line. This will toggle between run

and program load.

Example:

kit5k >dtr

dtr dropped on com 1

kit5k >dtr

dtr asserted on com 1

Dump  beg-addr, end-addr  >filename

Data is displayed as Intel Hex Data.  The optional arguments are

interpreted as hexadecimal  numbers as a default.  The beginning

and ending addresses are positional and will default to zero and

the current range  value if not  specified.  The  "greater than"

sign  signifies  that  output  is  to go  to  the specified file

instead of to the screen.

Example:

The following  command dumps the specified range of data to the

screen in Intel Hex format:

kit5K> du 0 2f

:2000000000010203FF05FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF FFFFFFFFFFFFF0

:10002000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFE0

:00000001FF

kit5K>

Alternatively, the command sequence  shown below will dump the

same data to a file called TEMP.HEX

kit5K> du 0 2f >temp.hex

kit5K>

Edit address

Data beginning with this address  is  available for examination

and change.   Data  may  be  examined  either in the forward or

backward direction. The argument is interpreted as a hexadecimal

number as a default.

Example:

kit5K> edit 0

X to exit edit; CR to leave unchanged; NUMBER to change.

0000  FF:00

0001  FF:01

0002  FF:02

0003  FF:

0004  FF:04

0005  FF:x

12

kit5K> display 0 f

0000  00 01 02 FF 04 FF FF FF - FF FF FF FF FF FF FF FF ................

kit5K>

Exit

Buffers and files are closed, the communication ports are cleared

and the program is exited.

Example:

kit5K> exit

A:\>

Fill value (beg-addr, end-addr)

The specified range  of locations is  filled with the specified

constant  value.   The  optional  arguments  are interpreted as

hexadecimal numbers as a default.

Example:

kit5K> fill ff 0 2f

kit5K> display 0 2f

0000  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0010  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0020  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

kit5K>

Help (command-name)

A list of available commands is listed or help on the requested

command is given.

Example:

kit5K> help load

name: load

function: Load the DS5000 memory from a file containing Intel

Hex data.

kit5K>

Key  (ASCII String | 5 pairs of hex digits | RANDOM)

This command sets the   Encryption  Key Word within the DS5000.

The  key  may  be  set  in  three  different  ways.  Five ASCII

characters enclosed  in single  or double quotes may be used to

specify the key.  The  user may request the computer to specify

five random numbers or the user may enter 5 hexadecimal numbers.

e.g.  KEY  'Stuff' , or KEY AA BB CC DD EE, or KEY RANDOM.

13

Example:

kit5K> key 0011223344

kit5K>

Load filename

The  data  from  the  filename  is  used  to program  the  chip.

Arguments  are  interpreted as hexadecimal numbers as a default.

Several  errors can be detected, such as multiple programming of

the same memory  location, and  attempts  to program nonexistent

memory.  All error  messages will be reported.  No configuration

data files are used  with this load.  See the description of the

Program command.

Example:

kit5K> load test.hex

kit5K> display 0 7f

0000  01 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00 ................

0010  00 00 00 00 00 00 00 00 - 00 00 00 00 00 00 00 00 ................

0020  54 45 58 54 20 46 4F 52 - 20 44 49 53 50 4C 41 59 TEXT FOR DISPLAY

0030  20 50 55 52 50 4F 53 45 - 53 2E 20 41 42 43 44 45 PURPOSES. ABCDE

0040  46 47 48 49 4A 4B 4C 4D - 4E 4F 50 51 52 53 54 55 FGHIJKLMNOPQRSTU

0050  56 57 58 59 5A 20 61 62 - 63 64 65 66 67 68 69 6A VWXYZ abcdefghij

0060  6B 6C 6D 6E 6F 70 71 72 - 73 74 75 76 77 78 79 7A klmnopqrstuvwxyz

0070  20 31 32 33 34 35 36 37 - 38 39 30 20 59 F6 41 11 1234567890 Y.A.

kit5K>

Lock

Set the Security Lock on the DS5000.

Example:

kit5K> lock

kit5K> status

Part is locked.

kit5K>

Logoff

The data logging buffers are flushed and the log file is closed.

Example:

(see the example given below for the Logto command)

14

Logto filename

Data logging is enabled to the specified file. Logging is turned

off with the Logoff command  or by  exiting.  All user input and

data echoed to the screen is logged.

Example:

kit5K> logto script

kit5K> fill ff 0 2f

kit5K> display 0 2f

0000  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0010  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

0020  FF FF FF FF FF FF FF FF - FF FF FF FF FF FF FF FF ................

kit5K> logoff

kit5K>

MCON number

The MCON Register  is  changed  to the  requested  number.  The

argument is interpreted as a hexadecimal number as a default.

Example:

kit5K> mcon 28

kit5K> status

Partition: 1000   Range: 8000 (32K)   Chip Enable 2: 0

kit5K>

Partition (0 | 800 | 1000 | 1800 | 2000 | 2800 | 3000 | 3800 | 4000 | 4800 | 5000

| 5800 | 6000 | 6800 | 7000 | 8000)

The Partition Address is set to the selected hexadecimal value

and the display is updated.  The  argument is interpreted as a

decimal number as a default.

Example:

kit5K> par 2000

kit5K> status

Partition: 2000   Range: 8000 (32K)   Chip Enable 2: 0

kit5K>

Out1

Toggles the status of the out1 signal on the PC/AT bus. This is

useful for PC based target  system. It  may  serve in a similar

manner to DTR.

Partition

Directly sets the partition as above

15

Pgmode

Signals the In System  Loader  hardware  to place the DS5000 in

Program  Mode  by  raising the  DTR  line  to its active level.

Communication  is then  attempted to the DS5000 at the selected

baud rate. The default value of the baud rate is 9600 bps if no

other baud rate frequency has been selected.

Example:

kit5K> pgmode

DTR asserted on com 1

kit5K>

Program filename [/edit]

The  data from the filename (default  extension = .HEX) and its

associated  configuration  file  (.CNF)  is used to program the

chip.   If the configuration file does not exist, the necessary

information will be prompted for  and  a new configuration file

will  be  created.  If  the  file  doesn't  exist  during batch

operation,  defaults will be used. The  configuration  file  is

displayed  before  the part  is  programmed.  The  /Edit switch

allows the current  configuration file to be edited one line at

a time.

Example:

(see Initial Checkout section)

Range  (2000 | 8000)

The range bit is set, and the display is updated.

Example:

kit5K> range 8000

kit5K> status

Partition: 2000   Range: 8000 (32K)   Chip Enable 2: 0

kit5K>

Reload

Sets the reload value of the PC 8250 UART. This is available for

more flexible selection of baud rates. The baud rate is selected

by the formula 115200/n where n is the reload number.

Example:

kit5k >reload 6

baud rate 19200

Run

The  DTR  line  is dropped to  its  inactive  level. This action

signals the In System Loader hardware to release the DS5000 from

its Program Mode and allow it to run the application program.

16

Example:

kit5K> run

DTR dropped from com 1

kit5K>

Speed (150 | 300 | 600 | 1200 | 2400 | 4800 | 9600)

This command is used to select the baud rate for communication

between the PC and the DS5000.  Decimal  numbers  are  used in

entering the arguments.

Example:

kit5K> sp 1200

kit5K>

Status

Display the current status of the DS5000.

Example:

kit5K> status

Partition: 2000   Range: 8000 (32K)   Chip Enable 2: 0

kit5K>

Term

Switch  to  terminal  emulation  mode. This  allows  testing  of

software  which  expects  serial  commands  from a PC or similar

station.  Many of the kit commands are available from this mode.

Online help is also available as with normal KIT mode.

Example:

kit5k> term

------- Dumb Terminal  Press Escape for Commands  ----------

"press escape"

dt> kit

kit5k>

Type filename

Type the requested file to the screen.

Example:

kit5K> type script

fill ff 0 2f

display 0 2f

kit5K>

17

Unasm

Unassemble the program RAM of the DS5000. Optional specify start

and stop addresses for disassembly.

Example:

kit5k> unasm 100 10C

0100 EB MOV A,R3

0101 F7 MOV @R1,A

0102 5191 ACALL 00291H

0104 B40A07 CJNE A,#00AH, 0010EH

0107 311F ACALL 0011BH

0109 20050F JB 005H,0011BH

010C 01DB AJMP 000DBH

KIT5K >

Unlock

Clear the Security Lock bit on the DS5000.

Example:

kit5K> unlock

kit5K>

Verify Filename  beg-addr, end-addr

The data  from  the filename is used to verify data in the chip.

The optional beginning and ending  addresses will determine what

parts of the input file will be used for verification. Data from

the  file  is compared against the current contents  of  memory.

Anytime  they  are different, the  discrepancy will be reported.

Arguments are entered as hexadecimal numbers.  The beginning and

ending addresses are positional and will default to zero  and to

the current range value if not specified.  No configuration data

files are used with this load.

Example:

kit5K> load test.hex

kit5K> verify test.hex

No Verification errors

kit5K

TERMINAL EMULATOR MODE COMMANDS

This  section  discusses  the  commands  which   are  unique  or

different in the terminal mode.  A complete list of commands for

this mode is given in a prior section.

Capture

When toggled on,  the  capture  command  receives  data from the

DS5000 and holds the contents into memory. When toggled off, the

program prompts for a filename to store to.

18

Example:

dt> capture

"an operation which sends data to the com port"

"escape"

dt> capture

Enter filename to save capture buffer, CR to discard.

Filename:

Help

Displays the commands available in terminal mode.

Example:

dt> help

Available commands:

CApture     CD          CLs         COm    DIR     DOS

DTr         EXit        Help        KIt    LOGOff  LOGTo

Nosnow      Out1        PGmode      Quit   REload  RUn

SEnd        SNow        SPeed       TYpe

Use "HELP command-name" or HELP *" for more information.

Kit

Switch out of terminal emulator into KIT.

Nosnow

Enable snow checking for CGA monitors. Screen output is slower.

Send

Sends  a file through a com port. Useful for serial download  to

an application program.

Snow

Disable snow checking on CGA monitors. Screen output is faster.

IN SYSTEM LOADER OPERATIONAL DETAILS

The  DTR  signal on the COM port interface is  activated  by  the

KIT5K  software to signal the hardware in the RS232 Fixture  that

serial downloading is in effect.   In addition,  RTS is activated

for the duration of the time that KIT5K is in execution.

When  DTR  is  activated,  the hardware isolates signals  on  the

DS5000 which are used to accomplish the serial download task from

the target system circuitry.  This includes RST,  TXD (P3.1), RXD

(P3.0),  and XTAL1.   RST is then driven high to initiate a reset

within the DS5000.   Following this action,  XTAL1 is then driven

with  the RS232 Fixture's 11.0592 MHz clock  oscillator  circuit.

Finally,  PSEN*  is  then driven low.   This sequence of  actions

causes  the DS5000 to begin operation in its Serial Program  Load

Mode  at  a  clock  frequency  from  which  standard  baud   rate

frequencies may be derived.

19

None  of the activity on the RST,  TXD,  RXD,  and XTAL1 pins  is

driven  out  to the target system lines while the DTR  signal  is

active.   DTR  will  remain active until either the "Run" or  the

"Exit" command is executed from KIT5K.

When DTR is released,  the In System Loader causes the DS5000  to

be reset and begin execution of the application code.   When this

occurs  and the In System Loader's switch is in the "A" position,

all communication performed on the TXD and RXD pins on the target

system is isolated from the PC.  If the switch is placed into the

"B" position,  then the TXD and RXD pins are connected to the  PC

and  not  to  the  target  system during  the  execution  of  the

application   program.    This   capability   is   provided   for

applications  which  require  that the  target  system  have  the

ability  to  communicate to the PC through the COM port  used  by

KIT5K.   In general, however, the "Run" command cannot be used to

initiate  the  execution  of DS5000  application  software  which

communicates  to  the  IBM  PC,  since KIT5K  will  still  be  in

execution and will have control of the COM port.

When operating the serial loader, the user should insure that the

port pins P2.7 and P2.6 are not externally pulled to a low state.

If this condition were to occur,  the Parallel Load Mode's Verify

cycle  would  be invoked.   As  a  result,  serial  communication

between  the KIT5K program on the PC and the serial loader on the

DS5000  would not function.   This is due to the fact that  since

RST  and  PSEN*  are being pulled high and  low  respectively  to

invoke the DS5000's Serial Load Mode operation,  the addition  of

P2.7  and P2.6 being pulled low would satisfy the condition for a

Verify cycle on the device.   The easiest way to insure that this

does  not occur is to assign either P2.7 or P2.6 as an output  in

the target system.   This will insure that at least one of  these

pins  will be pulled high by its internal pullup while the Serial

Load Mode is in effect,  as all port pins are set to a 1  (reset)

state during this time.

KIT5K OPERATIONAL DETAILS

When KIT5K is invoked from MS-DOS, communication is automatically

established  with  the  DS5000  via its  on-chip  Serial  Loader.

KIT5K always configures the COM port with 8 data bits, 1 stop bit

and no parity,  and activates RTS.  Communication with the DS5000

is then attempted by asserting DTR and sending a carriage  return

(<CR>) at the currently selected baud rate (default value is 9600

bps).   If no response is detected, then an error message to this

effect is displayed to the user.

After the baud rate is determined,  KIT5K holds both the DTR line

and  the RTS line at their active levels until either a "Run"  or

an  "Exit"  command  is  executed.   When the  "Run"  command  is

executed,  DTR  is taken inactive.   When the "Exit"  command  is

issued,  both DTR and RTS are de-asserted and control is returned

to the MS-DOS operating system.

20

DS5000T DEMONSTRATION SOFTWARE

The  DS5000TK  diskette  contains a set of  programs  which  work

together  to  allow  you to read and set the date and time  in  a

DS5000T  and  to compare it with the clock in your  IBM  or  IBM-

compatible PC.  The program DEMODS5T.HEX runs in the DS5000T, and

is  used to pass date and time information back and forth between

the  DS5000T  and  the PC over  the  serial  port.   The  program

DEMODS5T.EXE  runs in the PC,  and allows you to read or set  the

time in the DS5000T and compare it with the PC clock.

RUNNING DEMO.BAT

DEMO.BAT   automatically  loads  the  DS5000T  with   a   program

(DEMODS5T.HEX) to read the time and then runs a program in the PC

(DEMODS5T.EXE)  to  set  or  display the time sent  back  by  the

DS5000T. The following steps are required to run this demo:

A. Place  your  DS5000T in the zero insertion force socket  of  a

DS9060  RS232  Fixture and turn the socket lock screw  with  a

small screwdriver to the "C" position.   If you have an  older

RS232 fixture with a sliding socket lock mechanism,  press the

socket  lock firmly until the word "OPEN" and the heads of the

two  arrows  are clearly visible on the opposite side  of  the

socket.

B. Place  the  switch on the fixture in the position  labeled  B,

then  connect the fixture using the ribbon cable to  a  socket

providing  VCC  and Ground.   The socket must also  supply  an

11.0592 MHz crystal connected across pins 18 and 19, with pins

18 and 19 also each connected through a 30 pf capacitor to pin

20 (VSS).   Finally, the socket should insure that pin 31 (EA)

is tied to pin 40 (VCC).

C. Connect  the Fixture to the COM1 serial port of your PC  using

the  modular cable provided.   (Note that although this  cable

resembles an ordinary telephone cable, it is not the same, and

a telephone cable will not work properly.)

D. If  your PC does not have a built-in clock,  use the DATE  and

TIME  commands in DOS to set the correct date and time in your

computer.

E. Type  DEMO  on the keyboard to begin execution  of  the  batch

file.  This batch file first uses the KIT5K utility program to

load a program into the DS5000T which relays the date and time

information  back to the PC,  then it runs a program in the PC

which compares the time provided by the PC clock with the time

provided by the DS5000T.

F. Within  a few seconds,  the program should report that it  has

found the DS5000T,  and it will ask you if you want to set the

time.   If  you answer N,  the program will begin reading  the

date  and time information from the DS5000T and  comparing  it

with  the date and time from the PC clock.   If you answer  Y,

the program will set the date and time in the DS5000T to agree

with the PC clock, then it will begin comparing the two times.

The program will continue running until any key is pressed.

21

DEMODST.HEX OPERATION

DEMODS5T.HEX  is  the program that responds to commands  received

over  the  serial  I/O  port to send  or  receive  the  date/time

information  between  the embedded clock/calendar in the  DS5000T

and the serial I/O port.   This allows an external program access

to  the  date/time  information.   DEMODS5T.LST  is  an  assembly

language listing of this program.

The  program  first sets up the serial port for  transmission  at

9600  baud with eight data bits,  no parity,  and one  stop  bit.

Next,  the  program  begins  execution of a loop waiting  for  an

instruction from the serial port.   Two valid instructions, R and

W, are recognized. Receipt of the R character causes the DEMODS5T

program  to  read eight bytes of date/time information  from  the

embedded  clock/calendar and send them out over the serial  port.

Receipt  of the W character causes the DEMODS5T program  to  wait

for eight bytes of date/time information from the serial port and

write  them  to  the embedded  clock/calendar.   Any  other  byte

received  from the serial port is incremented and then sent  back

out to the serial port.

SAMPLE.HEX OPERATION

SAMPLE.HEX is a program which responds to commands received  over

the  serial  port  to set the date and time  information  in  the

DS5000T.   The  program  first  initializes the serial  port  for

communication at 9600 baud with eight data bits,  no parity,  and

one  stop  bit.   After setting the date and  time,  the  program

begins  execution of an infinite loop which sends back  the  date

and  time each time a character is received.   SAMPLE.SRC is  the

source  code  for  the program SAMPLE.HEX.   SAMPLE.LST   is  the

assembly language listing of the program SAMPLE.HEX.

22

Dallas  Semiconductor  Corporation Assumes No Responsibility  for

the  Use  of  Any Circuitry Other Than Circuitry  Embodied  in  a

Dallas Semiconductor Product.  No Other Circuitry Patent Licenses

Are implied.   Information Contained Herein Supercedes  Published

Specifications on These Devices from Dallas Semiconductor.

DALLAS SEMICONDUCTOR CORPORATION                      August 1988

4350 BELTWOOD PARKWAY SOUTH                              DS5000TK

DALLAS, TX  75244

(214) 450-0400

23

DS5000TK EVALUATION KIT OWNER REGISTRATION

Please  fill out the following form to receive software  upgrades

and future information on the DS5000TK Evaluation Kit and related

products.  Return the completed form to:

DS5000 Customer Service

Dallas Semiconductor

4350 Beltwood Pkwy

Dallas, TX  75244

Name ______________________________________________

Company ___________________________________________

Street_____________________________________________

Mail Stop _________________________________________

City______________  State _____  Zip ______________

Software Revision # _______________________________

Date of Purchase __________________________________

Place of Purchase _________________________________

DS5000 Application ________________________________

________________________________

Comments:  _____________________________________________

_____________________________________________

_____________________________________________